Abstract

Drowsy driving is one of the major causes of road accidents and death. Hence, detection of driver’s fatigue and its indication is an active research area. Most of the conventional methods are either vehicle based, or behavioural based or physiological based. Few methods are intrusive and distract the driver, some require expensive sensors and data handling. Therefore, in this study, a low cost, real time driver’s drowsiness detection system is developed with acceptable accuracy. In the developed system, a webcam records the video and driver’s face is detected in each frame employing image processing techniques. Facial landmarks on the detected face are pointed and subsequently the eye aspect ratio, mouth opening ratio and nose length ratio are computed and depending on their values, drowsiness is detected based on developed adaptive thresholding. Machine learning algorithms have been implemented as well in an offline manner. A sensitivity of 95.58% and specificity of 100% has been achieved in Support Vector Machine based classification.

Existing System

In existing system the driver drowsiness detection system involves controlling accident due to unconsciousness through Eye blink. Here one eye blink sensor is fixed in vehicle where if driver loses consciousness, then it alerts the driver through buzzer to prevent vehicle from accident. In future we can implement Drowsiness Detection System in aircraft in order to alert.

Proposed System

In Proposed System we develop a low-cost, real time driver’s drowsiness detection system with acceptable accuracy. Hence, we have proposed a webcam based system to detect driver’s fatigue from the face image only using image processing and machine learning techniques to make the system low-cost as well as portable the camera will be positioned in front of the driver to capture the front face image. From the Video, the frames are extracted to obtain 2-D images. From the facial landmarks, eye aspect ratio, mouth opening ratio and position of the head are quantified and using these features and machine learning approach, a decision is obtained about the drowsiness of the driver. If drowsiness is detected, an alarm will be sent to the driver to alert him/her. The details of each block are discussed below.

SYSTEM CONFIGURATION:

Hardware requirements:

Processer                     :           Any Update Processer

Ram                             :           Min 1 GB

Hard Disk                    :           Min 100 GB

Software requirements:

Operating System       :           Windows family

Technology                 :           Python 3.6

IDE			:	PyCham

UML			:	Star UML

DFD			:	DFD Drawer

Implemented by

Development team      :          Cloud Technologies

Website                       :           http://www.cloudstechnologies.in

Contact                        :           8121953811, 040-66411811